Linux Kernel Network Driver

alloc_netdev() Function Reference

dummy-eth Driver | Qualcomm QCS9075 | Kernel 6.6.116

# Overview

alloc_netdev() is the primary kernel function used to allocate and initialize a new network device structure (struct net_device). It must be called before registering a network interface with the kernel. The function combines memory allocation and initial setup into a single call.

# Function Signature

alloc_netdev(sizeof_priv, name, name_assign_type, setup);

In the dummy-eth driver this is called as:

alloc_netdev(0, "dummy%d", NET_NAME_UNKNOWN, dummy_setup);

# Argument Breakdown

| Argument | Value Used | Description |
| --- | --- | --- |
| sizeof_priv | 0 | Size of private driver data. 0 means no private state is needed since this is a dummy driver with no real hardware. |
| name | "dummy%d" | Name template for the interface. %d is auto-filled by the kernel, producing dummy0, dummy1, dummy2, etc. |
| name_assign_type | NET_NAME_UNKNOWN | Indicates how the interface name was assigned. Used by udev/systemd for predictable naming. UNKNOWN is appropriate for test/dummy drivers. |
| setup | dummy_setup | Function pointer called during allocation to initialize the net_device structure with ops, type, MTU, and MAC address. |

# Detailed Argument Explanation

## 1. sizeof_priv = 0

Specifies how many bytes of private data to allocate alongside the net_device. This memory is accessible via netdev_priv(dev) in the driver.

- Value 0 means the dummy driver has no hardware state to track

- A real driver (e.g. for a physical NIC) would pass sizeof(struct my_driver_priv)

- Private data holds things like register base addresses, DMA buffers, IRQ numbers

## 2. name = "dummy%d"

The name template for the network interface as it appears in the OS. The %d is replaced by the kernel with an incrementing integer.

- Results in interface names: dummy0, dummy1, dummy2, ...

- Visible via ip link show or ifconfig on the device

- Name templates follow standard Linux network naming conventions

## 3. name_assign_type = NET_NAME_UNKNOWN

Informs the kernel (and userspace tools like udev) about how the interface name was determined. This affects predictable network naming policies.

| Value | Meaning |
| --- | --- |
| NET_NAME_UNKNOWN | Name origin not specified. Suitable for dummy/test drivers. |
| NET_NAME_ENUM | Enumerated name (e.g. eth0, eth1). Assigned by the kernel in order. |
| NET_NAME_PREDICTABLE | Based on hardware topology (PCI slot, port, etc.). |
| NET_NAME_USER | Explicitly set by userspace (e.g. via ip link set name). |

## 4. setup = dummy_setup

A callback function invoked during alloc_netdev() to initialize the net_device fields. This is where the driver configures how the interface behaves.

Typical implementation for dummy-eth:

void dummy_setup(struct net_device *dev) {

dev->netdev_ops = &dummy_netdev_ops; // assign operations

dev->type       = ARPHRD_ETHER;      // Ethernet device type

dev->mtu        = ETH_DATA_LEN;      // MTU = 1500 bytes

eth_hw_addr_random(dev);             // assign random MAC

}

# Return Value

alloc_netdev() returns a pointer to the newly allocated struct net_device, or NULL if allocation fails.

struct net_device *dev = alloc_netdev(0, "dummy%d",

NET_NAME_UNKNOWN,

dummy_setup);

if (!dev)

return -ENOMEM;  // always check for NULL!

# Execution Flow

The following steps occur when alloc_netdev() is called:

- Kernel allocates memory for struct net_device + 0 bytes private data

- Interface name template "dummy%d" is stored; %d resolved on registration

- NET_NAME_UNKNOWN is recorded for udev/systemd naming policies

- dummy_setup(dev) is called to configure ops, type, MTU, and MAC address

- Pointer to net_device is returned to the caller

# After alloc_netdev()

Allocating the device is only the first step. The driver must then register it with the kernel to make it visible to the system:

ret = register_netdev(dev);

if (ret) {

free_netdev(dev);  // clean up on failure

return ret;

}

Once registered, the interface appears in ip link show as dummy0 and is ready for use. On module unload, always call:

unregister_netdev(dev);

free_netdev(dev);

dummy-eth Driver | QCS9075 Platform | Kernel 6.6.116 | Roopak 2026